AS1209: MATRIX COMPUTATION

Report on

Project: Solving Electrical Circuit using SVD based Matrix Computation

BY

Sarthak Pandit (2023Btech072)

JK Lakshmipat University, Jaipur

ODD 2025-26

Title:

Solving Electrical Circuit Using SVD Based on Matrix Computation

[b=Ax →V=IR]

Introduction:

Efficient numerical techniques are frequently needed to solve large circuits with numerous nodes and components in contemporary electrical engineering problems.
This project uses Singular Value Decomposition (SVD) to implement a circuit solver that can handle both consistent and inconsistent systems and analyse resistive networks.

The standard Ohm's Law serves as the foundation for the system:

V = I × R

Where:

V: Vector of voltage (known at nodes)

I: Current vector (problems to be solved)

R: Circuit connectivity resistance matrix

The project shows how SVD can calculate least-squares solutions for inconsistent systems and exact solutions for consistent systems

Methodology:

 Forming the Circuit Matrix:
Each node in the circuit is modeled, and the resistance matrix R is constructed.
Each equation corresponds to KCL/KVL constraints, forming the linear system:

A⋅I=V

Where A = R, I = currents, V = applied voltages

 Applying SVD:

- Decompose the matrix:

A=UΣVT

- U: Orthonormal basis of voltage space (nodes)

- V: Orthonormal basis of current space

- Σ: Singular values representing the stiffness of the circuit

 Solving the System:

- Consistent systems: I=A+V gives exact currents

- Inconsistent systems: I=A+V gives least-squares currents, minimizing the residual ∣V−AI∣

 Residual and Consistency Check:

- Residual vector: r=V−A⋅I

- Norm of residual: ∥r∥2​determines if the system is consistent or inconsistent

 Interpretation of Singular Values:

- Large singular values → stiff circuit (strong constraints, well-defined currents)

- Small singular values → near short-circuit or rank-deficient network (ill-conditioned)

Language Used:

- Python

Why SVD?

Typical linear solvers break when:

- Matrix is singular

- Equations are dependent

- Circuit is overdetermined (m > n)

- Circuit is underdetermined (m < n)

- System is inconsistent (no exact solution)

But SVD always works, because every matrix has a decomposition:

- A=UΣVT

The pseudo inverse is:

- A+=VΣ+UT

This gives a correct:

- Exact solution when consistent

- Least-squares solution when inconsistent

- Minimum-norm solution when underdetermined

This makes SVD the most stable and general-purpose method for solving circuits

Derivation:

Consider a resistive network
Using KCL at each node:

- ∑Iincoming=∑Ioutgoing

Using Ohm’s Law:

- I=(Vi−Vj)/Rij

Substituting currents into KCL gives a linear equation for each node:

- ∑(Vi−Vj)/Rij=Iinj

This produces a conductance matrix, which is your matrix A

Thus the equation becomes:

- A.I=V

So in my code:

- A → Resistances / conductances

- x → Currents (I)

- b → Applied voltages (V)

SVD Mathematical Interpretation:

SVD splits the matrix into three parts:

- A=UΣVT

### U (Voltage Modes): Columns represent voltage distribution patterns across the network.

### V (Current Modes): Columns represent independent current flow patterns.

### Σ (Singular Values): Represent the strength of each electrical mode:

- Small σ → Open Circuit (Low Current)

- Large σ → Short Circuit (High Current)

- Zero σ → Dependent equations → Infinite solutions

### Pseudo Inverse Formula:

- A+=VΣ+UT

And therefore:

### I=x=A+b

### A+ is the mathematical inverse of resistance matrix when A is not invertible

### Code:

### Output:

### The output for the Matrix taken from University of Florida:

### The output for the Matrix hardcoded in the code:

### Verification if the Answer Obtained by Code is Correct or Not:

Manual solution for m > n Inconsistent Matrix (A4 Matrix in code)

=== END OF REPORT ===